# Before You Begin

This manual shows you how to install the Microsoft® SQL Server Network Integration Kit for Novell® NetWare® networks on a server running the OS/2® operating system and on clients running the MS-DOS®, Microsoft Windows™, or OS/2 operating systems. It also describes how to use the Network Integration Kit to communicate with Microsoft SQL Server and applications for Microsoft Open Data Services using the native Novell Internet Package Exchange/Sequenced Package Exchange (IPX/SPX) protocol.

This manual is divided into four sections: “About the Network Integration Kit” introduces the Network Integration Kit; “Installing the Network Integration Kit” describes how to install the Network Integration Kit software on servers and clients; “Using the Network Integration Kit” describes how to start and stop the Network Integration Kit, and explains how to connect to a SQL Server from a client running Novell IPX/SPX; and “Troubleshooting” lists and describes error messages, and gives diagnostic information for connection failures.

## Document Conventions

This manual uses the following conventions to distinguish elements of text:

| Convention | Purpose |
| --- | --- |
| UPPERCASE | Represents Transact-SQL™ statements, paths, and filenames. |
| small caps | Represent names of keys, such as ctrl. |
| bold | Represents system procedures, utility programs, commands, and user-entered text. |
| italic | Represents database names, table names, and variables. |
| monospace | Represents examples, screen output, and error messages. |
| [brackets] | Enclose optional items. Type only the information within the brackets, not the brackets themselves. |
| ¦ (vertical bar) | Separates items inside a set of brackets. The vertical bar stands for “or,” and means you can choose only one of the items. |

This manual uses the term “OS/2” to refer to both the Microsoft Operating System/2® (MS® OS/2) and IBM® OS/2 systems.

## Finding Further Information

This manual contains all the information you need to install the Network Integration Kit on a server and on clients. For additional information about setting up clients and SQL Servers, see the following manuals:

Microsoft SQL Server Installation Guide

Provides comprehensive instructions for installing SQL Server software on clients and servers.

Microsoft SQL Server System Administrator’s Guide

Presents information about setting up and maintaining a SQL Server.

Microsoft Open Data Services User’s Guide

Provides information about using Microsoft Open Data Services.

## Obtaining Technical Support

<<This needs to be OK’d by support.>> Microsoft support for the Network Integration Kit is offered under the same programs as support for Microsoft SQL Server. For details, please see the support flyer included in the package. Outside the United States, contact your local Microsoft subsidiary. If you already have a support plan from Microsoft covering SQL Server, your existing plan will also cover the Network Integration Kit. Microsoft’s support services are subject to Microsoft’s prices, terms, and conditions in place in each country at the time the services are used.

# About the Network Integration Kit

The Microsoft SQL Server Network Integration Kit for Novell NetWare networks enables clients to communicate with Microsoft SQL Server and Microsoft Open Data Services using the Novell IPX/SPX protocol. The Network Integration Kit provides an excellent alternative to using Novell named pipes for communicating over a Novell network with a SQL Server.

In this manual, SQL Server is used to indicate both Microsoft SQL Server and Microsoft Open Data Services.

Note

The Network Integration Kit provides the following features and benefits:

u	It uses the IPX/SPX interface on both the client and the server. This means fewer software layers and easier installation and maintenance, and provides NetWare users the option of using a familiar protocol.

u	It eliminates the need for named pipes support software on MS-DOS– and Windows-based clients. This reduces memory requirements and provides fewer chances for terminate-and-stay-resident program (TSR) conflicts.

u	It works on clients that access both Microsoft- and Novell-based networks, such as clients running the Microsoft Windows for Workgroups operating system for integrated networks.

The Network Integration Kit includes both the client and server software required for clients running on a Novell NetWare network to communicate with Microsoft SQL Server. The Network Integration Kit software consists of:

u	An executable program and supporting libraries

u	Two server network libraries (also called Net-Libraries)

u	One client Net-Library for each supported client environment (MS-DOS, Windows, and OS/2)

u	Additional client support software, including DB-Library™ and IPX/SPX support for Windows

The executable program, called the Network Manager, passes network packets back and forth between Novell NetWare clients and a server running Microsoft SQL Server or Microsoft Open Data Services. The Net-Libraries, implemented as a TSR for MS-DOS – based clients and as dynamic-link libraries for the server and for Windows- and OS/2-based clients, perform the network operations

required for communicating between clients and a SQL Server. On the client, these Net-Libraries replace the named pipes equivalents that come with SQL Server applications.

To use the Network Integration Kit, you simply install the software on the server and on each client, and then start it by running a command file on the server.

# Installing the Network Integration Kit

Network Integration Kit installation is divided into two parts: installing the software on the server and installing the software on the clients.

Before you install the software, check that your system meets the configuration requirements, and then check your package contents and back up and register the software.

In addition to this manual, your Network Integration Kit package should contain the following items:

u	One 5.25-inch high-density (1.2-MB) Network Integration Kit Setup Disk

u	One 3.5-inch high-density (1.44-MB) Network Integration Kit Setup Disk

u	Network Integration Kit product registration card

u	Microsoft support services information

If you are missing any of these items, contact Microsoft Customer Service at (800) 426-9400. <<This needs to be OK’d by support.>> Outside the United States, contact your local Microsoft subsidiary.

Before you install the Network Integration Kit, make a backup copy of the Setup Disk by using the MS-DOS or OS/2 diskcopy command. For information about using diskcopy, see your operating system manuals.

In addition, be sure to mail in the product registration card to register your purchase of the Network Integration Kit with Microsoft. If you register your purchase, Microsoft will keep you informed of future releases.

## Installing the Server Software

You install the Network Integration Kit software on the server by:

u	Running the setup program (included on the Setup Disk).

u	Updating the command file (SQLSTART.CMD) used to start the Network Manager.

### Server Hardware and Software Requirements

To install the Network Integration Kit on the server, the server’s hard disk must have at least 1 megabyte (MB) of free space. In addition, you need the following hardware and software:

| You need | Your choices |
| --- | --- |
| Computer | Industry Standard Architecture (ISA) computer, such as the IBM PC/AT® and compatibles; Micro Channel® Architecture (MCA) computer, such as the IBM PS/2® and compatibles; or an Extended Industry Standard Architecture (EISA) computer with an 80286, 80386, or 80486 microprocessor running OS/2 version 1.3. An 80386 or 80486 microprocessor is recommended. |
| Minimum memory | 8 MB. |
| Disk drives | A minimum 40-MB hard disk and a 5.25-inch or 3.5-inch floppy disk drive. |
| Operating system | OS/2 version 1.3. |
| SQL Server software | Microsoft SQL Server version 4.2. |
| Network software | Novell NetWare Requester version NSD004 or later.1 |
| Network adapter | The appropriate network-adapter card.2 |
| 1 Install the Novell OS/2 Requester before you install the Network Integration Kit. 2 For information about the appropriate network-adapter card for your network, see your network manuals. |  |

### Running the Setup Program on a Server

The server installation option of the setup program copies the Network Manager executable file (NETMGR.EXE), supporting libraries for the Network Manager (NETCBP.DLL and NETMGRP.DLL), and the server Net-Libraries (SSMSSPXP.DLL and DBMSNP.DLL) to the appropriate directories on the server. It also updates the PATH and LIBPATH variables in the CONFIG.SYS file if necessary.

Û

To install the server software

1.	Insert the Setup Disk into drive A of the server.

2.	From an OS/2 command prompt, type:

a:setup

3.	Choose option 1 (Install the NIK server components for Novell).

4.	Follow the on-screen directions.

The setup program prompts you for the name of the root directory in which to copy the files. The default root directory is C:\SQL. If you install the software in a different directory, be sure to add the directory to your PATH. The setup program puts the Network Manager executable file (NETMGR.EXE) in a BINP directory under the root. It puts the libraries in a DLL directory under the root.

### Editing the SQLSTART.CMD Command File

The SQLSTART.CMD command file, copied to your BINP directory by the setup program, starts both SQL Server and the Network Manager. SQLSTART.CMD includes two commands:

start "SQL Server" /c sqlservr -dc:\sql\data\master.dat 
	-ec:\sql\log\errorlog
start "Network Manager" /c netmgr -NSSMSSPXP -SSQLSERVER 
	-Lc:\sql\log\netmgr.log

The first command starts SQL Server using C:\SQL\DATA\MASTER.DAT as the default MASTER database device and C:\SQL\LOG\ERRORLOG as the error log. The second command starts the Network Manager using SSMSSPXP as the server Net-Library, SQLSERVER as the service name, and C:\SQL\LOG\NETMGR.LOG as the log filename.

If the default arguments are not appropriate for your system (for example, if your MASTER database device is not C:\SQL\DATA\MASTER.DAT or if you want to use a different name for the service), you can update SQLSTART.CMD using any text editor. The service name must be unique on your NetWare network.

For additional information about starting the Network Manager, see “Starting the Network Manager,” later in this manual. For additional information about starting SQL Server, see the Microsoft SQL Server System Administrator’s Guide. For additional information about starting an Open Data Services application, see the Microsoft Open Data Services User’s Guide.

## Installing the Client Software

You install the Network Integration Kit software on a client by:

u	Running the setup program (included on the Setup Disk).

u	If the client is Windows- or OS/2-based, using either the w3dbver utility or the pdbver utility to indicate the appropriate Net-Library to use when communicating with the server.

### Client Hardware and Software Requirements

To install the Network Integration Kit on a client, the client’s hard disk must have at least 1 MB of free space. In addition, you need the following hardware and software:

| You need | Your choices |
| --- | --- |
| Computer | Industry Standard Architecture (ISA) computer, such as the IBM PC/AT and compatibles; Micro Channel Architecture (MCA) computer, such as the IBM PS/2 and compatibles; or an Extended Industry Standard Architecture (EISA) computer with an 80286, 80386, or 80486 microprocessor. An 80386 or 80486 microprocessor is recommended. |
| Minimum memory | 640K for MS-DOS- and Windows-based clients, and 3 MB for OS/2-based clients. |
| Disk drives | A hard disk and a 5.25-inch or 3.5-inch floppy disk drive. |
| Operating system | MS-DOS version 3.3 or later, Windows version 3.1 or later, or OS/2 version 1.3. |
| DB-Library software | DB-Library is included in the Network Integration Kit package. The Network Integration Kit supports any SQL Server 4.2 client.1 |
| Network software | For MS-DOS– and Windows-based clients, Novell IPX/SPX version 3.10 or later and NetWare Shell version 3.26 or later. MS-DOS– and Windows-based clients can also use the built-in NetWare support in Microsoft Windows for Workgroups. For OS/2-based clients, Novell NetWare Requester version NSD004 or later. |
| Network adapter | The appropriate network-adapter card.2 |
| 1 For MS-DOS-based applications, contact the application vendor regarding the availability of their product under DB-Library version 4.2. 2 For information about the appropriate network-adapter card for your network, see your network manuals. |  |

### Running the Setup Program on a Client

The client installation options of the setup program copy the client Net-Libraries into the appropriate directories on the client. If necessary, these options also update configuration information contained in the AUTOEXEC.BAT file (for MS-DOS- and Windows-based clients) and the CONFIG.SYS file (for OS/2-based clients).

Û

To install the client software

1.	Insert the Setup Disk into drive A of the client.

2.	From an operating system command prompt, type:

a:setup

3.	On MS-DOS –  or Windows-based clients, choose option 2 (Install the MS-DOS and Microsoft Windows client libraries). On OS/2-based clients, choose option 3 (Install the OS/2 client libraries).

4.	Follow the on-screen directions.

The setup program prompts you for the name of the root directory in which to copy the client files. The default root directory is C:\SQL. For MS-DOS- and Windows-based clients, the setup program puts the files in a BIN directory under the root. For OS/2-based clients, it puts the files in a DLL directory under the root.

The Network Integration Kit client software copied by the setup program includes the following files:

| File | Description |
| --- | --- |
| DBMSSPX.EXE | The IPX/SPX protocol Net-Library for MS-DOS-based clients. The setup program adds this TSR program to the client’s AUTOEXEC.BAT file. |
| DBMSSPX3.DLL | The IPX/SPX Net-Library for Windows-based clients. |
| DBMSSPXP.DLL | The IPX/SPX Net-Library for OS/2-based clients. |
| W3DBLIB.DLL | DB-Library version 4.2 for Windows-based clients. |
| PDBLIB.DLL | DB-Library version 4.2 for OS/2-based clients. |
| NWIPXSPX.DLL | The IPX/SPX support library for Windows-based clients. |
| NWNETAPI.DLL | The IPX/SPX support library for Windows-based clients. |
| W3DBVER.EXE | A utility for determining which version of DB-Library is installed on the client, and for setting the default Net-Library on the client. This utility is for Windows-based clients only. |
| PDBVER.EXE | A utility for determining which version of DB-Library is installed on the client, and for setting the default Net-Library on the client. This utility is for OS/2-based clients only. |
| ENDDBLIB.EXE | A program for MS-DOS-based clients that unloads the DBMSSPX.EXE TSR program. |

### Additional Setup for Windows-based Clients

If you’re using Windows-based clients to connect to a SQL Server using the Network Manager, you must update the client’s WIN.INI file to indicate the default Net-Library to use for connections. Update the WIN.INI file by using either the w3dbver utility (included on the Setup Disk) or a text editor. If you use w3dbver, set the default Net-Library to Novell IPX/SPX. If you use a text editor, the WIN.INI file entry should look like this:

[SQLSERVER]
DSQUERY=DBMSSPX3

Also verify that Windows is set up to run with the NetWare Shell version 3.26 or later. (The NetWare Shell version 3.26 is included with Windows 3.1.) You check your Windows setup by running the Windows Setup application. For additional information, see the NETWORKS.WRI file included with your Windows software.

### Additional Setup for OS/2-based Clients

If you’re using OS/2-based clients to connect to a SQL Server using the Network Manager, you must add an entry to the OS2.INI file to indicate the default Net-Library to use when communicating with the SQL Server. Update the OS2.INI file by using either the pdbver utility (included on the Setup Disk) or an OS2.INI file editor (such as writeini). If you use pdbver, set the default Net-Library to Novell IPX/SPX. If you use an OS2.INI file editor, the app_name is SQLSERVER, the key_name is DSQUERY, and the profile_string is DBMSSPXP.

# Using the Network Integration Kit

To use the Network Integration Kit, you start the Network Manager and SQL Server on the server, and then connect to the SQL Server from one or more clients.

The Network Manager can coexist with the named pipes support in the Novell OS/2 Requester, allowing both named pipes and IPX/SPX clients to simultaneously access a single SQL Server.

Note

## Starting the Network Manager

The recommended method for starting the Network Manager is by running the SQLSTART.CMD command file, located in the SQL\BINP directory on the server. The SQLSTART.CMD command file starts both SQL Server and the Network Manager. You can also start the Network Manager manually by typing the netmgr command from the OS/2 command prompt at the server. If you start the Network Manager manually, you must first start SQL Server.

The syntax for the netmgr command is:

netmgr -Nserver_netlib -Sservice_name [-Llog_file] [-palternate_pipe]
	[-salternate_socket]

where:

-Nserver_netlib

Indicates the name of the server Net-Library. The server_netlib should be SSMSSPXP, which is the default in the SQLSTART.CMD command file.

-Sservice_name

Indicates the name that the Network Manager should use when defining the SQL Server service to the native network naming service. The service_name will appear in the Novell Bindery databases on all file servers on the network. You use this name when connecting to the server from a client. For example, to connect from an isql client, type:

isql /Sservice_name /Usa /Ppassword

-Llog_file

Indicates the name of the log file. If log_file is not specified, the Network Manager writes the log to the file NETMGR.LOG in the directory in which the Network Manager was started.

-palternate_pipe

Indicates which alternate pipe to use to connect to the SQL Server. An alternate pipe is frequently needed to connect to an Open Data Services gateway, and is sometimes needed to connect to a SQL Server. The default pipe is \pipe\sql\query.

-salternate_socket

Indicates an alternate socket number to use when connecting to the Network Manager from a client. An alternate socket number is required if you need to connect to both a SQL Server and an Open Data Services gateway on the same computer. The default socket is 843E (hexadecimal). If you specify the socket number “0”, the Network Manager chooses a free socket.

## Network Manager Output

The Network Manager produces both screen output and log file output. The log file output is similar to the screen output, but the screen displays only connection information and the log file contains both connection information and error information.

### Screen Output

The Network Manager screen output, which the Network Manager displays in a window on the server, shows the service name and socket used by the Network Manager, and the pipe name for local connections. Here is an example:

Microsoft Network Manager version 4.20

Copyright (c) 1992, Microsoft Corporation

Network Manager Starting

Advertised Service Name = SQLSERVER

### Log File Output

The Network Manager log file (NETMGR.LOG) contains the same information as the Network Manager screen output, plus time stamps and details regarding error situations. Whenever you run into a problem running the Network Manager, the first thing you should do is check the log file.

Here is an example of the log file output for normal situations:

<9/4/92 15:15:16> Network Manager Starting

Advertised service name and socket = SQLSERVER,843E

Local SQL Server connection = \PIPE\SQL\QUERY

Here is an example of the log file output for error situations:

<9/4/92 15:26:30> SERVER THREAD ID: 4.

<9/4/92 15:26:30> SERVER PROCESS ID: 11760.

<9/4/92 15:26:30> SERVER: FATAL CONNECT ERROR: errornum = 26. 
 severity = 10, state = 0: Unable to close server-side connection.

For details about using the log file to diagnose problems, see “Troubleshooting,” later in this manual.

## Connecting to the Network Manager from a Client

If you’re connecting to the Network Manager from an MS-DOS– or Windows-based client, you must load IPX.COM and NETX.COM before starting the MS-DOS–based DB-Library application or Windows. In addition, if you want to run Windows in Standard mode, you must run TBMI2.COM prior to starting Windows. (For additional information, see the instructions in the NETWORKS.WRI file included with your Windows software.) The NetWare named pipes support (DOSNP.EXE) does not need to be loaded. In addition, clients do not need any special network permissions to access SQL Server via the Network Manager.

To access the Network Manager from a client, you simply connect to the SQL Server in the same way as you would ordinarily connect to a SQL Server, using the -Sservice_name argument to the netmgr command as the server name. For example, if you start the Network Manager using SQLSERVER as the -Sservice_name argument, you would use the following command to connect to the Network Manager from an isql client:

isql /SSQLSERVER /Ulogin_id /Ppassword

When using tools such as SQL Administrator or the System Administration Facility (SAF), the list of available SQL Servers will be empty. Type the name of the service as registered by the Network Manager on the server.

When using SQL Administrator from a Windows-based client, functions that use SQL Monitor require named pipes support on both the client and the server, so will not work with the Network Manager. Functions that use SQL Monitor include scheduling backups, viewing the error log, and viewing statistics.

Note

## Connecting More than 50 clients

The Network Manager is implemented as a single process, multithreaded application for OS/2 in which each client connection receives its own thread. Because of the thread-per-process limits in OS/2, the Network Manager spawns a second instance (process) of itself after 50 simultaneous clients are connected. The new instance is created in a new window, the Dynamic: Network Manager window, and the new instance has the same service name as the old. Client applications should see no difference in operation.

## Stopping the Network Manager

If you shut down the server from a client that is connected to the SQL Server through the Network Manager, the Network Manager stops automatically after the SQL Server shuts down. When you stop the Network Manager in this way, the Network Manager removes itself from the Novell Bindery and stops advertising itself as an available SQL Server. This is the preferred way to stop the Network Manager. You can also stop the Network Manager manually by selecting End Task from the OS/2 Task List on the server.

# Troubleshooting

The most likely problem you might encounter while using the Network Integration Kit will be an inability to establish a connection between a client and the Network Manager. If you’re having trouble establishing a connection, it’s usually the result of one or more of the following:

u	The Open Data Services application or the SQL Server has not been started.

u	The client is not a SQL Server 4.x client.

u	The SQL Server is not version 4.2.

u	The connection information used to start the Network Manager on the server is incorrect.

u	The connection information on the client (for example, the default Net-Library) is incorrect.

u	The required network hardware connections between the computers have not been established.

u	The required network software has not been installed or is incorrectly installed on one or both sides of the connection.

## Error Messages

The Network Manager alerts you to connection problems by writing error messages to one or more of the following:

u	The client

u	The Network Manager window

u	The log file

### Error Messages Written to the Client

The Network Manager writes the following error messages to the client when there is a problem making a connection:

Login failed. Network Manager only supports 4.x clients.

Indicates that the client is not supported by the Network Manager. These are the requirements:

·	For MS-DOS–based clients, you must use a version that has been linked with DB-Library version 4.2.

·	For Windows- and OS/2-based clients, you must use the Microsoft SQL Server version 4.2 named pipes Net-Library and DB-Library DLLs.

Unable to connect: Network Manager is unable to establish a connection with the server.

Indicates that the Network Manager is available but can’t connect to the SQL Server. For help in determining the cause of the failure, see “Isolating a Problem,” later in this manual.

If the client is unable to establish a connection with the Network Manager, DB-Library writes the following error message to the client:

DB-Library: Unable to connect: SQL Server is unavailable or does not exist.

Indicates that DB-Library could not find a Network Manager service under the name specified. For help in determining the cause of the failure, see “Isolating a Problem,” later in this manual.

### Error Messages Written to the Network Manager Window

If the Network Manager cannot be started, it writes the following message to the Network Manager window:

Unable to initialize Network Manager. Check log file.

Indicates a problem with your configuration, or with network hardware or software. Check the log file for a more detailed error message that can help you isolate the problem.

### Error Messages Written to the Log File

The Network Manager writes one or more messages to the log file when it encounters an error. Detailed error codes and time stamps are also written to the log file to aid in troubleshooting.

The following errors prevent the Network Manager from starting properly:

Unable to load client-side Net-Library

Indicates that the Network Manager was unable to load the Net-Library for the connection between the Network Manager and the SQL Server (DBMSNP.DLL). The Net-Library is either nonexistent or not in the path.

Unable to load server-side Net-Library

Indicates that the Network Manager was unable to load the Net-Library for the connection between the client and the Network Manager (SSMSSPXP.DLL). The Net-Library is either nonexistent or not in the path.

Unable to set up server-side connection

Indicates that the Network Manager was unable to create a socket on the server. This error occurs if there is a problem with the network (such as not enough resources).

The following errors can occur occasionally when handling client requests. They generally affect only a single client connection and do not stop the SQL Server or the Network Manager:

Unable to read from server-side connection

Indicates that the Network Manager was unable to retrieve a request from the client. This error message is sent if a network error occurs while the client is sending a request to the Network Manager. It can also occur if the client breaks the connection before it has finished sending a request to the Network Manager.

Unable to write to server-side connection

Indicates that the Network Manager was unable to send results from the Network Manager to the client. This error message is sent if a network error occurs while the Network Manager is sending results to the client. It can also occur if the client breaks the connection before the Network Manager has finished sending results to the client.

Unable to close server-side connection

Indicates that the Network Manager was unable to close the connection between the client and the server.

## Isolating a Problem

If you’re having problems establishing or maintaining connections, check the following items.

### Problems Establishing Connections

The following list describes things you should check when you’re having trouble establishing a connection between a client and the Network Manager. Check:

1.	The version numbers of the network components.

The version numbers of the network components should match the requirements specified in “Installing the Network Integration Kit,” earlier in this manual.

2.	The version of the DB-Library software on the client.

The DB-Library software on the client must be version 4.2 or later. You can check the version of the client by running w3dbver on Windows-based clients and pdbver on OS/2-based clients. For MS-DOS–based clients, DB-Library is linked in, so you must get the appropriate version from the application vendor.

3.	The connection between the client and the network.

The client must be connected to the network. If it’s connected, you’ll get the message “Attached to server server_name” when you boot the client computer (for OS/2-based clients) or after NETX.COM is loaded (for MS-DOS– and Windows-based clients).

4.	The connection between the server and the network.

The server must be connected to the network. If it’s connected, you’ll get the message “Attached to server server_name” when you boot the server.

5.	The configuration of the client.

The appropriate Net-Library must be loaded on the client. For MS-DOS – based clients, check that the DBMSSPX TSR program has been loaded (not the DBNMPIPE TSR). For Windows- and OS/2-based clients, check that the default Net-Library is set to DBMSSPX3 or DBMSSPXP, respectively.

6.	The parameters used to start the Network Manager.

The -Nserver_netlib argument must be SSMSSPXP, and the -Sservice_name argument must match the service name requested from the client.

7.	The local connection to the SQL Server using named pipes.

You should be able to make a local connection to the SQL Server using isql (with no server_name argument) and named pipes as the default Net-Library. If you can’t make a local connection using named pipes, there’s a problem with the SQL Server.

8.	The local connection to the SQL Server using the service name.

You should be able to make a local connection to the SQL Server using isql (with the /S argument set to the service_name used when starting the Network Manager) and DBMSSPXP as the default Net-Library. (Note that you must change the default network value between steps 7 and 8). If you can do step 7 but not step 8, the Network Manager is not registered properly on the network.

9.	The network connection between the client and the server.

If you can do steps 7 and 8, but you can’t connect from a client across the network, check that both the client and the server can connect to a file server on the local area network where the SQL Server is located. A failure to connect to a common file server indicates a routing or configuration problem with your network.

Problems Maintaining Connections

If you’re having trouble with connections being dropped or client applications “hanging” when using the Network Manager, this usually indicates a network error. Problems can occur when there are no free network buffers to service requests, or when there are conflicts over shared network or memory resources. For current information about optimum network parameter settings, see the README.NIK file installed in the Network Integration Kit root directory (typically C:\SQL).